COMPUTER ORGANIZATION AND ASSEMBLY LANGUAGE

BCS-III(B)

PROJECT REPORT

Project Title:

Logical Expression Evaluator

Submitted To:

Majid Shafique

Group Members:

Hajra Bashir (036)

Haniya Atiq (054)

1. Project Overview:

The Logical Expression Evaluator is an assembly language–based program developed using 8086 architecture. The project evaluates logical expressions entered by the user and also demonstrates real-life applications of logic using predefined scenarios. It provides a menu-driven interface and displays step-by-step logical calculations.

2. Logical Expression Evaluator Description:

The evaluator allows users to input logical expressions using binary values (0 and 1) and logical operators such as AND (&), OR (|), NOT (!), and XOR(^). It parses the input expression, validates it, performs logical operations, and displays the final result as TRUE or FALSE.

3. Features of the Code:

- Menu-driven user interface

- Supports basic logical operators (AND, OR, NOT, XOR)

- Handles complete expressions with parentheses

- Input validation (accepts only 0 or 1)

- Step-by-step calculation display

- Real-life logic simulations (Elevator & Bank Vault systems)

- Clear screen and reusable procedures for modularity

4. Architecture of the Code:

The code follows a procedural and modular architecture:

- Main Module: Displays main menu and controls program flow

- Simple Evaluator Module: Parses and evaluates logical expressions

- Scenario Modules: Implements elevator and bank vault logic

- Utility Procedures: Input handling, output display, screen clearing

Each module is implemented as a separate procedure to improve readability and reuse.

5. Working of Logic Gates in the Code:

Logical gates are implemented using 8086 bitwise instructions:

- AND gate → AND instruction

- OR gate → OR instruction

- NOT gate → NOT followed by masking (AND 1)

- XOR gate → XOR instruction

These instructions operate on binary values (0 or 1) to simulate digital logic behavior.

6. Working of Registers in the Code:

- AX, AL: Used for calculations and intermediate results

- BL, BH, CL, CH, DL: Store temporary logical values

- SI, DI: Used for input buffer handling

- DX: Used for displaying strings

Registers efficiently store operands and results during logical evaluations.

7. Flowchart:

- Start – Program execution begins.

- Display Main Menu – User sees options: Simple Evaluator, Real-Life Scenarios, Exit.

- User Choice – Program checks the selected option.

- Simple Evaluator Path

- User enters a logical expression

- Expression is parsed and validated

- Logical operations are evaluated

- Result (TRUE/FALSE) is displayed

- Real-Life Scenarios Path

- User selects a scenario

- Inputs (0 or 1) are taken

- Logic gates are applied step-by-step

- Final decision (Access/Door Open or Denied) is shown

- Exit – Program terminates.

8.Logics & Truth Tables:

8.1 Elevator Door Safety System

- Logic:

Door Opens = <<Floor & <!Obstruction>> & <Button | Timer>> & <!Emergency>

- Truth Table:

Inputs:

- Floor (F): Elevator aligned with floor

- Obstruction (O): Obstacle detected

- Button (B): Close button pressed

- Timer (T): Timer expired

- Emergency (E): Emergency stop active

Output:

- Door (D): 1 = Door opens, 0 = Door remains closed

| F | O | B | T | E | Door (D) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 | 0 |

- Explanation:

The door opens only when the elevator is aligned, no obstruction exists, either button or timer is active, and no emergency is triggered.

8.2 Bank Vault Access System

- Logic:

Access Granted= <ManagerVote | <CEO & <!Alert>>> & <Normal ^ Emergency>

- Truth Table:

Inputs:

- M1, M2, M3: Manager approvals

- CEO: CEO approval

- Alert: Security alert

- Normal: Normal hours & physical key

- Emergency: Emergency hours & biometric

Output:

- Access (A): 1 = Access Granted, 0 = Access Denied

| ManagerVote | CEO | Alert | Normal | Emergency | Access |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 |

- Explanation:

Access is granted only when authorization is valid (managers or CEO without alert) and exactly one mode (Normal or Emergency) is active.

9. Conclusion:

This project successfully demonstrates how logical expressions and digital logic systems can be implemented using 8086 assembly language. It enhances understanding of logic gates, registers, and low-level program control while linking theory with practical real-life scenarios.

________________